# CA Lab 2 — Testing & Branching (Redesigned with Solutions, Workflows & Diagrams)

This Lab 2 integrates the provided handout and sample codes, expands the problem set, and adds step-by-step workflows, pseudocode, and runnable solutions for QtSpim. Topics: pseudo‑instructions, conditional branching, loops, and string processing.

## 1. Overview & Goals

After this lab, students can: (1) expand MIPS pseudo‑instructions to real instructions; (2) use bne/beq/bgt/bge/blt/ble for control flow; (3) build input loops and exit flags; (4) manipulate strings in memory.

References: Patterson & Hennessy; QtSpim/SPIM docs; course Lab 02 guide.

## 2. Pseudo‑instructions (li) → Real MIPS Instructions

Given sample: Lab2_2_1.s writes a value to $t0 and prints it.

# From Lab2_2_1.s
# li $t0, 5      # pseudo
# Real form when immediate fits 16-bit signed:
addiu $t0, $zero, 5

Change 1: li $t0, -5  → expands to:

addiu $t0, $zero, -5   # -5 fits 16-bit signed immediate

Change 2: li $t0, 0xaabbccdd  → expands to:

lui  $t0, 0xAABB
ori  $t0, $t0, 0xCCDD
# Note: if the lower 16-bit has sign-bit set and assembler cares about sign, it may use addiu in second step;
# SPIM commonly uses 'lui' + 'ori' for 32-bit constants.

Workflow: Identify immediate size → If |imm| ≤ 2^15−1 use addiu; else use lui+ori.

## 3. Branching Game — Guess the Number

Starter given: Lab2_3_1.asm. Secret = 0x1FA (decimal 506). The sample always prints 'lose' because control falls through after WIN.

# Fix: jump over LOSE after printing WIN
bne  $t0, $t1, LOSE
WIN:
  # print win
  # ...
  j   END
LOSE:
  # print lose
END:
  jr $ra

3.3 One‑shot compare (no iteration): print '<' / '=' / '>' using slt/bgt/bge/blt/ble.

# Lab2_3_3.s (one-shot)
# assume t0=secret, t1=guess
beq  $t0, $t1, EQUAL
bgt  $t1, $t0, HIGHER     # guess > secret
# else guess < secret
# print "Too low"
j END
HIGHER:
# print "Too high"
j END
EQUAL:
# print "Correct"
END: jr $ra

3.4 Repeat until correct: loop with read→compare→branch; break on equal.

# Lab2_3_4.s (loop until correct)
LOOP:
  # prompt & read guess -> t1
  beq  $t0, $t1, WIN
  blt  $t1, $t0, TOOLOW
  # else too high
  j LOOP
WIN:
  # print "You win"
  j END
TOOLOW:
  # print "Too low"
  j LOOP
END: jr $ra

3.5 Add exit flag: after each attempt, ask user 1=continue, 0=stop.

# Lab2_3_5.s (loop with exit flag)
LOOP:
  # prompt & read guess -> t1
  beq  $t0, $t1, WIN
  blt  $t1, $t0, LOW
  # HIGH
  # print "Too high"
  j ASK
LOW:
  # print "Too low"
ASK:
  # ask 'continue? 1/0' -> v0
  beq  $v0, $zero, END
  j LOOP
WIN:
  # print 'win'
END: jr $ra

## 4. String — Capitalize First Letter of Every Word

Read into buffer with syscall 8; then scan. Uppercase first alphabetic after space/start; leave others unchanged.

# Lab2_4_solution.s
.data
buf:    .space 200
sp:     .asciiz " "
nl:     .asciiz "\n"
prompt: .asciiz "Enter a line: "
.text
.globl main
main:
  # prompt
  la  $a0, prompt
  li  $v0, 4
  syscall

  # read string
  la  $a0, buf
  li  $a1, 200
  li  $v0, 8
  syscall

  # transform: capitalize first letter of each word
  la  $t0, buf          # ptr
  li  $t7, 1            # at_word_start = 1
loop:
  lb  $t1, 0($t0)
  beq $t1, $zero, done

  beq $t7, $zero, not_start
  # if at start of word and 'a'..'z' -> to 'A'..'Z'
  li  $t2, 'a'
  li  $t3, 'z'
  blt $t1, $t2, not_start
  bgt $t1, $t3, not_start
  addi $t1, $t1, -32    # to upper
  sb  $t1, 0($t0)
not_start:
  # update at_word_start: true if current is space or newline or tab
  li  $t4, ' '
  beq $t1, $t4, set_start
  li  $t4, '\n'
  beq $t1, $t4, set_start
  li  $t4, '\t'
  beq $t1, $t4, set_start
  li  $t7, 0
  j next
set_start:
  li  $t7, 1
next:
  addi $t0, $t0, 1
  j loop

done:
  # print result
  la  $a0, buf
  li  $v0, 4
  syscall

  la  $a0, nl
  li  $v0, 4
  syscall

  li  $v0, 10
  syscall

## 5. Hidden Challenge — Word/Digit/Upper/Lower Counters

Count words, digits, lowercase, uppercase in the input and print the statistics.

# Lab2_hidden_stats.s
# counts words (space-delimited), digits '0'..'9', lowercase, uppercase
# ... (similar scanning as above; maintain counters in $s*

## 6. Workflows & ASCII Diagrams

Guessing Game (3.4/3.5):

[Start] -> [Prompt & Read Guess] -> [Equal?] --yes--> [Print WIN] -> [End]
                                   --no--> [Too low/high message] -> [Continue? 1/0]
                                              |1 -> back to [Prompt & Read Guess]
                                              |0 -> [End]

String Capitalization (4):

[Read buffer] -> [i=0, at_start=1] -> loop:
  ch = buf[i]
  if ch == 0: print & exit
  if at_start and 'a'<=ch<='z': ch-=32
  at_start = (ch in {' ', '\n','\t'})
  i++

## 7. Step‑by‑Step Checklist for Students

1) Load .s file → 2) Run → 3) Single‑step to observe registers → 4) Set breakpoints near branches → 5) Verify console outputs → 6) Annotate memory addresses where applicable.

## 8. Solutions Summary

• Pseudo‑instr expansions: addiu for small signed; lui+ori for full 32‑bit.

• Secret number: 0x1FA = 506; fix fall‑through by jumping past LOSE after WIN.

• One‑shot compare prints: Too low / Correct / Too high.

• Loop until correct with optional exit flag.

• Capitalize first letter of each word by tracking at_word_start.

## References (Scholarly Style)

Patterson, D. A., & Hennessy, J. L. (2014). Computer Organization and Design: The Hardware/Software Interface (5th ed.). Morgan Kaufmann.

Larus, J. R. (2004). Assemblers, Linkers, and the SPIM Simulator. University of Wisconsin–Madison.

Lab 02 handout and sample sources (International University – VNU HCM, 2023).