Laboratory 3 – ITITIU22240 – Đàm Nguyễn Trọng Lễ

Exercise 1:  Write a program that use only

and	andi	nor	or 	ori	sll	srl	xor	xori

  - Put the number 0xABC22240 into register $t1 without using pseudo-instruction li. (lab4_1_1.s)

Code:

.data

.text

.globl  main

main:

andi    $t1,    $t1,    0x0     #clear the value of t1

ori     $t1,    $t1,    0xABC2  #load the upper 16 bits to t1

#t1=0x0000ABC2

sll     $t1,    $t1,    16      #shift the value of t1 to the left by 16 bits

#t1=0xABC20000

ori     $t1,    $t1,    0x2240  # or bit the lower 16 bits of t1, 0x2240

#t1=0xABC22240

jr      $ra

Explain:

1st: We use andi $t0 to make sure that we return $t1 value to 0
2nd. Load the first 16 bits of 0xABC22240 (0xABC2) to $t1 by LOGIC OR
3rd. Then shift 16 bits left to OR with 16 bits left (0x2240)

QTSPIM”

  - Redo 1.1 as follows: use ori to load each letter into register. (lab4_1_2.s)

Code:

.data

.text

.globl  main

main:

and     $t1,    $t1,    $zero   # Clear $t1

ori     $t1,    $t1,    0xA

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0xB

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0xC

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0x2

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0x2

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0x2

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0x4

sll     $t1,    $t1,    4

ori     $t1,    $t1,    0x0

#printing the value of $t1

li      $v0,    1

move    $a0,    $t1

syscall

# Exit the program

li      $v0,    10

syscall

Explain:

With same method to problem 1.1 but rather than each step ORI we use full (short) 16 bits we just use 4 bits = 1 digits of hexadecimal, thus after each ORI we shift left 4 bits

QTSPIM:

After add the 0xA

  - Suppose that $t1 = 0xABC22240. Using only register-to-register logic and shift instructions, Reverse the order of the bytes in $t1 so that register $t2 get the bit pattern 0x04222CBA  (lab4_1_3.s)

Code:

.data

.text

.globl  main

main:

and     $t1,    $t1,    $zero       #clear the value of t1

and     $t2,    $t2,    $zero       #clear the value of t2

and     $t3,    $t3,    $zero       #clear the value of t3

ori     $t1,    $t1,    0xABC2

sll     $t1,    $t1,    16

ori     $t1,    $t1,    0x2240

andi    $t3,    $t1,    0xF         # mask the lower 4 bits of t1 => t3(temporary register) for storing the last 4 bits of t1

or      $t2,    $t2,    $t3         # or bit the last 4 bits of t1 to t2

sll     $t2,    $t2,    4           # shift t2 left by 4 bits

srl     $t1,    $t1,    4           # shift t1 right by 4 bits

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

andi    $t3,    $t1,    0xF

or      $t2,    $t2,    $t3         # or bit the last 4 bits of t1 to t2

srl     $t1,    $t1,    4           # clear the last 4 bits of t1

srl     $t3,    $t3,    4           # clear t3

jr      $ra

Explain:

After we set $t1 = 0xABC22240 and set $t2, $t3 = 0, we:

- Mask the last 4 bits by ANDI $t1 with 0xF and store the result into temp register($t3)

- Then we use OR to store the result into last 4 bits of $t2

- Then we shift left $t2  4 bits to hold the place for the incoming value

- After that we shift right $t1  4 bits to prepare the newest last digit(hex) for the loop

- Repeat until we get all the digits in $t1 (hexadecimal)

QTSPIM:

Set $t1 = 0xABC22240

After the 1st iteration:

Result: R10 [t2] = 4222cba (the last digits is 0 => the result only 7 rather than 8 digits in total)

  - Redo 1.3 using only and, or, and rotate instructions. (lab4_1_4.s)

Code:

.data

.text

.globl  main

main:

lui     $t1,    0xABC2

ori     $t1,    $t1,    0x2240

ori     $t4,    $zero,  0xF

and     $t2,    $t2,    $zero       #clear the value of t2

and     $t3,    $t3,    $zero       #clear the value of t3

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3

sll     $t2,    $t2,    4

srl     $t1,    $t1,    4

and     $t3,    $t1,    $t4

or      $t2,    $t2,    $t3         # or bit the last 4 bits of t1 to t2

srl     $t1,    $t1,    4           # clear the last 4 bits of t1

srl     $t3,    $t3,    4           # clear t3

jr      $ra

Explain:

We use the same method with 1.3, except the load to $t1, $t4 we use ori, all the line left is free of addi and ori.

QTSPIM:

Exercise 2:  Write a program that

  - Set the corresponding bit in register $t1 through $t8. That is, in register $t1 set bit 1, register $t2 set bit 2, and so on. (lab4_2_1.s)

Code:

.data

.text

.globl  main

main:

ori     $t1,    $t1,    0x1     # Load the value 0x1 into register $t1

sll     $t2,    $t1,    1       # Shift the value in $t1 left by 1 bit and store it in $t2

sll     $t3,    $t2,    1

sll     $t4,    $t3,    1

sll     $t5,    $t4,    1

sll     $t6,    $t5,    1

sll     $t7,    $t6,    1

sll     $t8,    $t7,    1

# Exit the program

li      $v0,    10

syscall

Explain:

The idea behind this is simple, we store the value 0x1 into $t1 by using ori. Then we using shift left 1 bits to act as activation for bits in 2nd position in $t2. After that we treat the $t2 as cascading step to activate the bits in 3rd position in $t3. Looping until we set the bits in 8th position in $t8 on.

QTSPIM:

  - By using ONLY shift instructions and register to register logic instructions (no li pseudoinstruction or addi), put the pattern 0xFFFFFFFF into register $t1. (lab4_2_2.s)

Code:

.data

.text

.globl  main

main:

ori     $t1,    $t1,    0x1     # Load the value 0x1 into register $t1

sll     $t2,    $t1,    1       # Shift the value in $t1 left by 1 bit and store it in $t2

or      $t3,    $t2,    $t1     # t3 = 0x0011 = 0x3

sll     $t4,    $t3,    2       # t4 = 0x1100 = 0xC

or      $t5,    $t4,    $t3     # t5 = 0x1111 = 0xF

sll     $t6,    $t5,    4       # t6 = 0x11110000 = 0xF0

or      $t7,    $t6,    $t5     # t7 = 0x11111111 = 0xFF

sll     $t8,    $t7,    8       # t8 = 0x1111111100000000 = 0xFF00

or      $t9,    $t8,    $t7     # t9 = 0x1111111111111111 = 0xFFFF

#clear the value of t1, t2, t3, t4, t5, t6, t7, t8,

and     $t1,    $t1,    $zero

and     $t2,    $t2,    $zero

and     $t3,    $t3,    $zero

and     $t4,    $t4,    $zero

and     $t5,    $t5,    $zero

and     $t6,    $t6,    $zero

and     $t7,    $t7,    $zero

and     $t8,    $t8,    $zero

# put 0xFFFFFFFF in t1

or      $t1,    $t1,    $t9

sll     $t1,    $t1,    16

or      $t1,    $t1,    $t9

# Exit the program

li      $v0,    10

syscall

Explain:

First, we set the $t1 = 0b1 then we use sll 1 bit to get $t2 = 0b10

We or $t1 and $t2 to get 0b0011 and store that in $t3, then sll $t3 2 bit and store that in $t4 to get 0b1100 , then we or $t3 and $t4 to get 0b1111 and store that in $t5

Then we repeat the same method to $t6 , $t7 , $t8 , $t9 to get 0xFFFF

Then we clear $t1 -> $t8 and apply the method we use in 1.1 to store the 0xFFFF in upper half 16 bits and lower half 16 bits of $t1

QTSPIM:

We got the $t9 = 0xFFFF